# Lossless Audio Compression with Huffman Coding

Programming Assignment 1 — Track B
Video Link: https://youtu.be/0gsAxcXjuzs?si=ufDXQEvm2exQNCmo

# (a) What I built

For this assignment, I implemented Huffman coding and developed a custom lossless audio compression pipeline. The resulting tool reads raw PCM WAV files, encodes audio streams into a specialized binary container, and decodes the stream back into a WAV file that is byte-for-byte identical to the original input.

The codec is written entirely in Python without relying on external signal processing, FFT, or compression libraries. Python standard library wave module handles WAV file parsing and I/O, while all underlying components, including the priority queue, binary tree, bit packing, and container structure, were constructed from scratch. Modules such as numpy and pytest are used exclusively for verification and unit testing.

## Adapting textbook Huffman coding to audio

The canonical algorithm calculates symbol frequencies, pushes nodes onto a priority queue, repeatedly merges the two lightest nodes under a new parent, and derives variable-length codes from root-to-leaf paths. I implemented this core structure using a custom binary min-heap rather than Python's built-in heapq module, preserving the exact O(n log n) construction runtime and ensuring that only the sort key is ever compared, never the tree nodes themselves.

Applying the algorithm to audio, though, forced four changes on me.

Applying this algorithm directly to audio signals required four critical architectural modifications:

Canonical codes. Rather than serializing and transmitting the full tree hierarchy, the codec transmits only each leaf's depth and reconstructs codes canonically. Codes are assigned by sorting symbols by length and value, assigning consecutive integers within each length, and bit-shifting left when moving to the next length. This technique, mirroring the DEFLATE specification, provides two distinct advantages. First, the decoder requires only a {symbol: length} mapping (approximately 800 lengths per typical block) instead of an entire serialized tree. Second, it resolves tie-breaking ambiguities. When node weights match, varying heap extraction orders alter tree shapes; canonical assignment guarantees identical code reconstruction regardless of encoder tie-breaking choices.

Length-limited codes. Highly skewed symbol distributions can produce codes exceeding the stream container's 20-bit limit. When encountered, the encoder halves all symbol frequencies (enforcing a minimum count of 1) and rebuilds the tree. Flattening the distribution balances the tree depth, guaranteeing termination because uniform weights yield a balanced tree of depth ⌈log₂ n⌉. While slightly suboptimal compared to the package-merge algorithm, this weight-halving heuristic provides a reliable, low-complexity solution.

Prediction before coding. Huffman coding requires skewed symbol frequencies to achieve compression. Uncompressed 16-bit PCM audio exhibits near-uniform sample distributions, measuring roughly 11.9 bits of entropy per sample against a 16-bit ceiling. To generate the necessary skew, samples are transformed into prediction residuals. Order 0 retains raw samples, whereas order k calculates successive sample differences k times. Because adjacent audio samples are highly correlated, residuals cluster tightly around zero. Each block evaluates prediction orders 0 through 3 and selects the option that minimizes bit consumption.

Adaptive block sizes. Every block carries its own codebook, meaning larger blocks amortize header overhead across more samples. Conversely, smaller blocks adapt more effectively to localized signal transitions because each block uses a single prediction mode and code table. To optimize this trade-off, each 65,536-sample superblock is recursively split in half down to 1,024 samples, retaining splits only when the combined size of child blocks is smaller than the parent block. Split decisions are encoded into the bitstream using a preorder tree representation.

## Representing the data

## Representing the data

Residual values are signed, whereas Huffman symbols must be non-negative. To keep small magnitudes near zero, the encoder applies a Zigzag mapping:

0, -1, 1, -2, 2, ...  ->  0, 1, 2, 3, 4, ...

Following Zigzag transformation, symbols cluster densely around zero. Storing the table as delta gaps between consecutive sorted symbols ensures most gaps equal 1, which encodes into single bits under Elias gamma coding. Code lengths are stored as relative changes from adjacent symbols and compressed using a small secondary Huffman table, taking advantage of the fact that neighboring symbols share similar code lengths. This secondary coding reduces codebook entry costs to one or two bits per symbol instead of a fixed five-bit representation.

## How the code is organised

## Codebase architecture

The architecture enforces strict single-direction module dependencies:

cli.py          compress / decompress / stats

|

codec.py        prediction, block splitting, mode choice, container format

|     \

huffman.py      tree, canonical codes, encode, decode     audio_io.py   WAV

|

bitio.py        BitWriter / BitReader

huffman.py operates completely independent of audio domain logic, processing generic hashable symbols. Audio prediction, block partitioning, mode selection, and container formatting are isolated within codec.py. Core Huffman coding mechanisms are centered in huffman.py, while top-level module documentation in codec.py outlines the binary container specification.

# (b) The tool

## What it does

## Command-line interface and subcommands

ahuf is a command-line lossless audio compressor featuring three dedicated subcommands:

python cli.py compress   input.wav  output.ahuf [--block-size N] [--min-block-size N]

python cli.py decompress input.ahuf output.wav

python cli.py stats      input.wav  [--blocks]

compress and decompress operate as inverse functions, verified by comparing MD5 checksums of original and reconstructed audio streams. The stats subcommand analyzes compression metrics without generating output files, serving as the primary analytical tool for this report.

## Where Huffman sits in the pipeline

## Positioning Huffman coding in the execution pipeline

Huffman coding acts as the entropy coding stage at the foundation of the pipeline, with all preceding stages preparing optimal symbol distributions:

WAV samples

-> split into blocks (adaptively)

-> for each block, for each prediction order 0..3:

compute residuals -> zigzag -> count frequencies

-> build Huffman tree -> canonical codes -> measure exact size

-> keep whichever of {verbatim, constant, order 0..3} is smallest

-> write mode, table, and codes to the bitstream

A key design choice is that prediction modes are evaluated by fully encoding every candidate mode and measuring exact bit outputs rather than relying on theoretical entropy estimations. Theoretical estimates ignore codebook table overhead, which can comprise up to 43% of short block data. Comparing exact encoded sizes guarantees optimal configuration selection for every block.

A dedicated VERBATIM mode stores raw fixed-width samples when compression provides no benefit. This mode places a strict ceiling on bitstream expansion, limiting maximum frame expansion to the 3-bit mode descriptor and enabling white noise signals to compress at 1.0002× original size rather than expanding.

## Interface decisions

## Interface design and error handling

The commands compress, decompress, and stats are structured as distinct subcommands rather than flags. Separating mutating tasks from read-only inspections prevents accidental file overwrites during analysis.

stats functions as a dedicated diagnostic command because encoding runs at roughly 56,000 samples per second. It provides reports detailing residual entropy per prediction order, achieved bits per sample, codebook table costs, and block mode distribution. This breakdown identified that unoptimized block sizes were introducing an 18% overhead penalty.

Block sizes are configurable via --block-size and --min-block-size. While ideal settings depend on input material, default parameters of 65,536 and 1,024 samples were established through empirical benchmark evaluations.

Error handlers provide clear diagnostic feedback. For example, unsupported 32-bit floating-point WAV exports are intercepted with an explicit explanation recommending re-export as 16-bit or 24-bit PCM, avoiding cryptic low-level system errors like unknown format: 3.

The file header includes an MD5 checksum of raw audio samples. This checksum guards against bitstream corruption that might otherwise decode silently into invalid audio. Across eight test corruption scenarios, structural validation caught seven cases; the eighth, a zeroed file tail, decoded 20,000 corrupted samples silently until the checksum verification was added.

## Worked example

$ python cli.py stats tones.wav

1 channel(s), 66150 frames, 16-bit, 22050 Hz, 2 blocks

entropy, order 0 residual:  11.88 bits/sample

entropy, order 1 residual:  10.92 bits/sample

entropy, order 2 residual:  11.80 bits/sample

entropy, order 3 residual:  10.97 bits/sample

achieved (incl tables):      11.48 bits/sample vs 16 uncompressed

code tables:                  0.49 bits/sample

audio by mode:              order1 100%

Order 1 prediction achieves the lowest residual entropy (10.92 bits), causing all blocks to select it. Order 2 prediction yields higher entropy than Order 1 because double differencing overshoots signal trends on this material. The achieved encoding rate of 11.48 bits per sample sits 0.56 bits above the Order 1 entropy baseline. Code tables account for 0.49 bits of this delta, with the remainder resulting from Huffman coding constraints regarding fractional bit lengths.

$ python cli.py compress tones.wav tones.ahuf

132344 -> 94935 bytes (71.7% of original)

$ python cli.py decompress tones.ahuf back.wav

wrote 1 channel(s), 66150 frames, 16-bit, 22050 Hz

$ md5 -q tones.wav back.wav

e8f31ec9268639e7fa52d0df05747417

e8f31ec9268639e7fa52d0df05747417

Identical MD5 checksums confirm that the encoding and decoding cycle is completely lossless.

## Results

The codec was benchmarked against standard general-purpose compression tools using 80,000-sample synthetic test signals (160,000 bytes of 16-bit PCM). Output sizes are listed in bytes:

| signal | mine | gzip -9 | bzip2 -9 | xz |
| --- | --- | --- | --- | --- |
| 80 Hz with vibrato | 24,145 | 156,585 | 141,886 | 139,120 |
| plucked notes | 25,009 | 147,688 | 55,819 | 35,096 |
| sine + inaudible dither | 79,241 | 158,303 | 141,342 | 98,592 |
| quiet noise | 92,991 | 117,490 | 97,532 | 105,724 |
| white noise | 160,025 | 160,068 | 161,089 | 160,068 |
| chord (exact harmonics) | 24,807 | 18,566 | 29,927 | 14,224 |
| pure sine (exact period) | 25,107 | 5,696 | 9,233 | 4,320 |

The custom audio codec wins or ties against general-purpose compressors on signals featuring realistic variation, falling behind only on synthetic, perfectly periodic waveforms, as detailed in section (c).

To measure the contribution of adaptive block splitting, each signal was evaluated with adaptive splitting disabled:

| signal | splitting on | off | gain |
| --- | --- | --- | --- |
| tone → silence → noise | 93,846 | 111,094 | 15.5% |
| fast alternation | 173,474 | 189,615 | 8.5% |
| note attacks | 35,294 | 37,584 | 6.1% |
| stationary tone | 36,723 | 36,723 | 0.0% |

Efficiency gains are highest where signal characteristics transition rapidly. On stationary tones, the partitioning logic correctly opts not to split blocks.

# (c) What I learned

## Inaudible input variation shifted relative performance roughly ninefold

This finding was unexpected initially, though logical in retrospect.

The custom codec underperformed gzip on a pure 80 Hz sine wave, yielding 25,107 bytes versus 5,696 bytes. To analyze this gap, ±40 LSB of dither was added to the signal. This random noise was inaudible at 16-bit resolution, barely affecting the overall sample value distribution while destroying exact sequence repetition entirely.

|  | mine | gzip |
| --- | --- | --- |
| pure 80 Hz sine | 25,107 | 5,696 |
| same + ±40 dither | 79,241 | 158,303 |

Output from gzip expanded 28-fold, increasing from 5,696 bytes to 158,303 bytes, whereas the custom codec expanded by roughly 3-fold. This swung the relative performance ratio between the two codecs roughly ninefold (from 4.41× to 0.50×). This difference occurs because gzip combines LZ77 dictionary matching with Huffman coding. LZ77 relies on discovering identical byte sequences. An 80 Hz sine wave sampled at 44.1 kHz repeats every 551 samples, allowing LZ77 to match long sequences. Dither disrupts exact sequence matching while leaving the underlying probability distribution virtually intact.

This experiment demonstrates that Huffman coding and LZ77 leverage different forms of signal redundancy. Huffman coding targets skew in the value distribution, while LZ77 targets sequence repetition. General-purpose compression can be poorly matched to audio because it often looks for sequence repetition rather than distribution skew. While dictionary matching excels on perfectly periodic synthetic tones, real-world music is rarely periodic enough for LZ77 sequence matching to be effective. Adjacent samples contain high correlation that linear predictors can extract, which explains why dedicated formats like FLAC rely on predictive modeling rather than general-purpose dictionary compression.

## Differencing doesn't always help

## Differencing does not universally reduce entropy

A common assumption is that delta encoding consistently improves audio compression. Measuring entropy before and after differencing reveals a more nuanced reality:

| signal | raw | delta | change |
| --- | --- | --- | --- |
| chord, three low tones | 12.85 b | 9.67 b | −3.19 |
| bass tone 80 Hz | 11.09 b | 9.01 b | −2.08 |
| mid tone 1 kHz | 8.78 b | 7.79 b | −1.00 |
| white noise | 14.73 b | 14.91 b | +0.18 (worse) |

On white noise, differencing increases entropy. Calculating differences between independent random samples increases distribution variance, widening symbol spread. Similarly, higher prediction orders degrade compression on non-smooth signals, causing Order 2 prediction to underperform Order 1 in the worked example. Evaluating prediction modes per block based on exact encoded size ensures optimal selection across varying signal types.

## The code table can cost more than the data

## Codebook overhead can exceed payload data size

Prior to implementing adaptive block sizing, analyzing block costs revealed that codebook headers comprised 13.8% of a standard block and 42.8% of short terminal blocks. On one test file, two out of nine blocks reverted to VERBATIM mode because table overhead negated coding gains.

This reveals a core challenge in per-block Huffman coding for audio: alphabet size scales alongside block size. An 8,192-sample block contained approximately 1,080 distinct residual symbols, requiring a thousand table entries to encode eight thousand samples. Increasing the block size from 8,192 to 65,536 improved compression efficiency by 8.7% on mono and 18.6% on stereo files without modifying the underlying algorithm.

This dynamic explains why dedicated codecs rarely transmit full per-block Huffman tables for residuals. FLAC utilizes Rice coding, which parametrizes distributions using a single integer per block, accepting slightly lower coding efficiency per symbol to eliminate table overhead. The two-level table encoding implemented here, which Huffman-codes the code lengths themselves, reduced table overhead to 0.49 bits per sample.

## What I got wrong along the way

Several of my most useful lessons came from mistakes.

## Engineering pitfalls and lessons learned

Several valuable insights emerged from addressing development errors:

Tie-breaking. The first version used Python's built-in heapq, pushing (weight, node) tuples. That works until two weights are equal: heapq compares tuples element by element, so on a tie it falls through to comparing the Node objects themselves, finds no ordering defined for them, and raises a TypeError. This is part of why the final implementation uses a hand-written binary min-heap instead — it compares only the key and never the item attached to it, so the situation cannot arise.

The crash turned out to be hiding a more serious problem. When two weights tie, the shape of the tree is no longer determined by the frequencies alone; a different tie-break produces different code lengths from identical input. Since compression and decompression are separate processes, any disagreement between them desynchronises the bitstream and produces plausible but incorrect audio with no error raised. Canonical coding is what actually resolves this: because codes are derived from the stored lengths by a fixed rule, both sides reconstruct identical codes regardless of how the encoder broke its ties.

Unverified default parameters. Block size was originally set to 8,192 samples without empirical verification. Combined with fixed 5-bit length storage, this setting caused a 9% to 19% loss in compression efficiency. Systematic profiling across block sizes prompted the development of two-level code length encoding and adaptive partitioning.

Silent uncompressed fallbacks. At small block sizes between 512 and 2,048 samples, every block defaulted to uncompressed storage because table overhead exceeded coding gains. The fallback executed correctly, preserving data at 16.00 bits per sample, but reported successful compression without indicating that no data reduction occurred. Detailed status reporting was added to expose these silent fallbacks.

Invalid test validation. The header checksum exists to catch corrupted stream tails. Initial tests passed, but further inspection revealed that an upstream range check was rejecting invalid inputs before checksum validation occurred. The test suite was updated to break specific pipeline components and confirm that individual guards trigger as intended.

Circular test dependencies. A failed byte-for-byte round-trip test was initially resolved by re-saving input files through the custom WAV writer. This masked underlying I/O discrepancies by comparing the custom writer against itself. Tests were subsequently updated to perform direct sample-level comparisons against raw source audio.

Incomparable performance metrics. A performance review indicated a decoding time of 1.44 seconds for ten seconds of audio, whereas earlier benchmarks measured 0.3 seconds. Running both evaluations through a unified test harness revealed the cause: the reviewer's test signal contained dither, producing 4,428 unique residual symbols versus 984 and tripling compressed data size. Because decode time scales with table complexity and stream volume, throughput measured 328,000 versus 2,832,000 samples per second. Both measurements were accurate, but differed due to input signal characteristics.

These experiences reinforced that theoretical assumptions must be continually validated through empirical execution and testing.

# (d) Use of AI

## Tool allocation and workflow

Claude (via Claude Code) was used throughout development alongside a second Claude instance acting as an independent reviewer. AI assistance was utilized across specific domains:

For conceptual background and explanations, AI tools assisted in reviewing Huffman coding proofs, canonical code assignments, and entropy bounds in prediction modeling. Automated scripts were also used to collect empirical measurements.

The implementation involved shared contribution. I guided core algorithm design, verified data structures, and resolved pipeline defects. AI generated portions of the boilerplate code and helper routines. The audio_io.py module was adapted from a prior project, modified to keep audio samples as raw integers rather than normalized floating-point values.

The test suite was generated primarily by AI. Manual auditing later identified two defective test cases passing due to invalid assumptions, highlighting the necessity of verifying auto-generated tests.

For code review and profiling, Claude evaluated implementation passes to identify bottlenecks, leading to insights regarding block-size overhead, gzip behavior, and dither sensitivity. A second AI instance reviewed these findings, and all recommendations were manually verified before integration.

Manual testing was performed independently. Manual execution revealed edge cases such as silent fallbacks during small block sizes, which automated unit tests did not capture.

This report was drafted by Claude using benchmark data and notes, then edited for clarity and structure.

## Where the AI got things wrong

## AI output limitations and patterns

Defects encountered in AI-generated code and test cases fell into three recurring patterns:

First, unverified default values. Parameters like the 8,192-sample block size and fixed 5-bit length storage appeared reasonable initially but degraded compression efficiency. Systematic benchmark profiling across parameters resolved these defaults.

Second, tests incapable of failing. Auto-generated tests occasionally asserted trivial conditions, such as checking isinstance(value, float) on a .real property, which always evaluates to true. Tests were audited by intentionally breaking underlying logic and confirming test failures. The final test suite explicitly verifies errors across eight distinct bitstream corruption scenarios.

Third, unreported fallback behaviors. Small-block fallbacks executed silently without warning that compression was bypassed, requiring explicit diagnostic logging to surface runtime decisions.

I fixed every defect described in this section. Several were discovered directly through manual testing, while others emerged from the code review process; each issue was individually verified before making code changes.

Review scripts occasionally misread internal specifications, such as assuming read_wav returned normalized floats in [-1, 1] rather than signed integers. Similarly, a dead-code scanner incorrectly flagged active functions as unused due to accounting errors in usage counts. These cases highlighted the necessity of developer oversight when interpreting automated tools.

## What I understand, and what I took on trust

## Technical understanding and verified concepts

I have a complete understanding of the core system components, including Huffman tree construction and greedy choice optimality proofs; prefix-free properties and tree leaf decodability; canonical code assignment for tie-breaking elimination; Zigzag mapping and Elias gamma coding; bitstream manipulation in bitio.py; predictive residual transform orders; and dither impact on entropy coding.

Specific auxiliary domain conventions were adopted based on established specifications:

- Kraft inequality validation in Decoder.__init__, verifying that code length tables describe valid prefix-free trees without overlapping paths.

- Weight-halving heuristics for length-limited codes, providing a low-complexity alternative to the package-merge algorithm.

- Maximum code length bounds set to 20 bits, matching standard DEFLATE stream conventions.

- Low-level WAV container parsing in audio_io.py, verified through losslessness testing across 8-bit, 16-bit, 24-bit, and 32-bit PCM audio inputs.

- Recursive half-splitting search, demonstrated empirically to improve compression on dynamic signals by up to 15.5% while preserving efficiency on stationary tones.

- Automated unit test coverage, where critical execution paths were manually audited and verified by introducing deliberate faults.

# References

- Huffman, D. A. (1952). "A Method for the Construction of Minimum-Redundancy Codes." Proceedings of the IRE, 40(9), 1098–1101.

- Cormen, T. H., Leiserson, C. E., Rivest, R. L., & Stein, C. Introduction to Algorithms, 3rd ed., §16.3 (Huffman codes and the greedy-choice proof).

- Deutsch, P. (1996). DEFLATE Compressed Data Format Specification version 1.3, RFC 1951. Canonical code construction and code-length coding.

- Coalson, J. FLAC Format Specification. https://xiph.org/flac/format.html. Fixed predictors, block structure, and the decoded-audio MD5.

- Robinson, T. (1994). SHORTEN: Simple lossless and near-lossless waveform compression. Cambridge University Engineering Department Technical Report 156. Prediction followed by entropy coding for audio.

- Salomon, D. Data Compression: The Complete Reference, 4th ed. Length-limited Huffman codes and the weight-halving heuristic.